[image: image1.wmf][image: image2.wmf]            SafetyWorks! Toolbox Talks

Subject: Ladders
Injuries in the workplace because of ladder are commonplace. Falls from ladders can be as painful as a fall from a roof; about a third of all reported falls are falls from ladders. Many of the fall related injuries result from the improper use or the use of a defective ladder. Step/extension ladders are made to access/egress upper levels, not to be used as work platforms. There are specifically designed ladders for use as work platforms such as order pickers. These ladders are constructed with a small platform and guardrail. The following safe work rules should be observed when working with ladders.
Guide for Discussion 
Inspection 
· Look for missing or loose cleats at the bottom. 

· Look for loose or missing screws, bolts or nails on job made ladders 

· Look for cracked, broken, split, dented or badly worn rungs, cleats or side rails. 

· Splinters on wood ladders. 

· Corrosion on metal ladders. 
Ladder Use 
· Always use the right ladder for the right job. 

· Don’t set your ladder in a walkway or door opening. 

· Keep the area at the top and bottom of the ladder clear of tool cords, tools, material and garbage. 

· Always set the ladder on solid footing. 

· Use a twenty-five percent (4:1)  angle on the slope of the ladder. 

· When using extension ladders, the three (3) top rungs must extend beyond the landing platform. (Or the top of an extension ladder must be 36” (3 feet) above the landing. 

· Don’t lean to the side when on a ladder or you may tip over. 

· Do not carry tools or materials on a ladder. Use both hands when climbing a ladder to grab onto the side rails. If it is necessary to move material or tools up a ladder, first climb up, then pull up the work with a hand line. 

· Only one person on a ladder at a time (unless the ladder is double cleated). 

· Always secure the top of the ladder to prevent it from sliding. 

· Never lean a step ladder; always fully open a step ladder. 

· Always face the ladder. 
Additional Discussion Notes: 

Always tie off the ladder. That way it stays where you put it.
Remember: When you are on a ladder, you can fall. If you can fall, you can get hurt. Use ladders safely.
NOTE: Always promote a discussion on any of the topics covered in the Tool Box Talks. Should any question arise that you cannot answer, don’t hesitate to contact your Employer or SafetyWorks!. 
Subject:  Ladders
Meeting Conducted By: _____________________________  Date: ________________

Attendees

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

________________________________     ___________________________________

For additional Information or scheduling a FREE consultation, call 623-7900 or 

Toll FREE (877) SAFE345

Keep this training record and subject matter in the site office with the site manager!
SafetyWorks! ( 45 State House Station ( Augusta, Maine 04333

Phone: (207) 623-7900

Website: www.safetyworksmaine.com


